Bike Lane Network Gap Analysis

New York City  ·  QGIS Portfolio Project 03

| Tools | QGIS 3.34, QNEAT3 plugin, MapInfo Pro (cross-check), Python 3.11 (geopandas, networkx) |
| --- | --- |
| Data Sources | NYC Open Data (DOT Bike Routes), ACS 2022 5-Year Estimates, OpenStreetMap |
| CRS | NAD83 / New York Long Island (EPSG:2263) |
| Status | Complete |
| Repo | github.com/[your-username]/nyc-bike-network-gap-analysis |

## Overview

Network connectivity analysis of NYC's protected bike lane infrastructure. Identifies disconnected segments, low-coverage areas relative to population density, and priority corridors for network expansion based on demand weighting (population density and commute data).

Core question: which parts of the city have high commuter populations but poor protected bike access — and which existing gaps, if closed, would have the greatest impact on network reach?

## Data Sources

| Dataset | Source | Format | Notes |
| --- | --- | --- | --- |
| NYC Bicycle Routes | NYC Open Data (DOT) | Shapefile | Protected lanes + greenways; facility_t IN (1, 2) |
| ACS 2022 5-Year | US Census Bureau | CSV | Tables B08301 (commute mode) + B01003 (population) |
| Census Tract Boundaries | TIGER/Line 2022 | Shapefile | NYC 5-borough extent |
| OSM Road Network | OpenStreetMap via QuickOSM | GeoPackage | Network routing backbone |

## Methodology

### 1. Network Topology Audit

After filtering to 623 protected/greenway features (from 1,847 total), ran QGIS Topology Checker with three rules: no dangles, no self-overlaps, no duplicate geometries. Found 47 dangling endpoints and 12 isolated subnetworks. Corrected 16 snapping errors (gaps < 5m); 31 were legitimate endpoints.

### 2. Gap Identification

Gaps defined as OSM road segments where protected lanes exist on both approach ends but not on the segment itself, length < 500m. Found 94 candidate gaps totalling 11.2 km, using QNEAT3 ISO area generation from network endpoints.

### 3. Demand Scoring

demand_score = (pop_density_norm    * 0.50)
             + (bike_commuter_norm  * 0.30)
             + (gap_length_inv_norm * 0.20)

### 4. Coverage Analysis

Percentage of each census tract's population within 400m network distance of a protected bike lane, calculated using QNEAT3 ISO areas (run by borough due to memory limits on the full 5-borough network).

## Key Findings

- 12 isolated subnetworks — the protected lane network is not a single connected graph.

- 94 gap candidates identified; top 3: 180m gap on Flatbush Ave (Brooklyn), 220m gap on 9th Ave (Manhattan), 310m gap on Queens Boulevard.

| Borough | Coverage (%) | Notes |
| --- | --- | --- |
| Manhattan | 71.4% | Best coverage — dense network along west side |
| Brooklyn | 48.2% | Moderate; strong core, weak outer areas |
| Queens | 31.7% | Large area, sparser network |
| Bronx | 22.1% | Significant unmet demand vs ACS commuter data |
| Staten Island | 8.3% | Lowest coverage — isolated from main network |

## Troubleshooting Log

### QNEAT3 memory crash on full city network

Full NYC OSM network (~280,000 segments) caused out-of-memory error. Fix: split by borough, ran separately, merged results. Runtime per borough: 4–12 minutes.

### GEOID format mismatch — ACS join

~40% NULL values after ACS join. Cause: NYC Open Data GEOIDs omit the state FIPS prefix (036). Fix: Field Calculator expression lpad() to reconstruct full 11-digit GEOID. NULL rate dropped to < 1%.

### Topology false positives — dual-classified lanes

3 'duplicate geometry' errors were two lanes sharing the same physical path (e.g., protected lane + greenway classification). Not errors — added as exceptions in the topology report.

## Skills Demonstrated

- Network topology audit: dangling endpoints, duplicate geometries, snapping error correction

- Connected component analysis (graph theory via networkx)

- QNEAT3 ISO area (service area) generation — borough-level parallel processing

- Multi-criteria demand scoring with normalized weights

- ACS census data join and GEOID field format troubleshooting

- MapInfo Pro cross-validation of topology results

Data: NYC Open Data, US Census Bureau, OpenStreetMap contributors — all publicly available. Code: MIT License. Map outputs: CC BY 4.0.